Monesh R

PALO ALTO

INITIAL CONFIGURATION:

Steps

- Login to MGMT

MGMT IP: 1.0.0.10                                                                                            SUBNETMASK: 255.0.0.0

- Next Login to Palo alto

- Assign ip address for management pc and go to GUI of paloalto

PALO IP: 1.0.0.1                                                                                            SUBNETMASK: 255.0.0.0

Now go to run on windows enter ncpa.cpl

Set the IP, subnet mask and default gateway

- Now enter IP address – https://1.0.0.1

- Go to network create a zones

In this Zones create Inside and outside zone

- Go to Interfaces and set Set IP address, Default router, zone to create interfaces

- For Ethernet 1/1 and 1/2

- Add default route to ISP

Select Virtual routers set static routers

Click default in Static routers eth1/1 and eth1/2

Give name: Default

- Select Object and set address

- We want to create DNS is important

- Create policy for inside and outside

- We want to create NAT policy

- Enable DHCP for lan interface

- After configuration finished we want to give commit

- Now ping 8.8.8.8  from LAN